Topic: Mean, Median and Gaussian Filter

Tripti Vishwakarma

Colorado State University Global

Course Code CSC 515

Dr. Jonathan Vanover

June 7th, 2025

Title: Comparative Analysis of Mean, Median, and Gaussian Filters for Impulse Noise Removal in Digital Images

This report evaluates the performance of three image filtering techniques—Mean, Median, and Gaussian filters—applied to an image corrupted with impulse noise (commonly referred to as salt-and-pepper noise). The filters were applied using three different kernel sizes (3x3, 5x5, and 7x7) to observe the effects of kernel size on noise removal and feature preservation. Additionally, Gaussian filters were tested with two Sigma values (σ=1 and σ=2) to investigate the influence of Gaussian smoothing parameters on the output.

Impulse noise manifests as randomly distributed black and white pixels, significantly degrading an image's quality. The primary challenge lies in removing this noise while maintaining important features such as edges, textures, and fine details. This report aims to identify the most effective filter and configuration for impulse noise removal based on a detailed analysis of visual results. The findings are supported by theoretical insights and references from existing literature.

Preferred Filter for Impulse Noise Removal

Impulse noise is best addressed using the Median filter, as it is explicitly designed to handle outliers in the pixel intensity values. The Median filter works by replacing each pixel value with the median of its surrounding neighbors within the kernel. This method effectively removes noise without compromising edge sharpness, as the median operation inherently ignores extreme values (such as the black and white pixels typical of impulse noise). In contrast, the Mean filter averages all pixel values within the kernel, including noisy pixels, resulting in blurred edges. Similarly, Gaussian filters, while effective for removing Gaussian noise, struggle to handle impulse noise because their weighted averaging mechanism still incorporates noisy pixel values in the computation.

Farid (2004) highlights the robustness of the Median filter in suppressing high-amplitude noise such as salt-and-pepper noise. The author explains that the Median filter’s ability to disregard extreme outliers makes it particularly effective in preserving edges and fine details. Bhuyan (2019) further supports this claim, noting that Median filtering minimizes edge distortion compared to other techniques such as Mean or Gaussian filtering. These theoretical insights align with the practical results observed in this study, where the Median filter consistently outperformed the other filtering methods in removing impulse noise.

Best Visual Performance

The visual results clearly indicate that the Median filter with a 3x3 kernel provides the best balance between noise removal and feature preservation. For this configuration, the noise was effectively removed, and the edges and fine details of the image remained sharp and intact. Larger kernel sizes, such as 5x5 and 7x7, resulted in slightly improved noise reduction but introduced minor blurring, particularly in textured areas. This tradeoff between noise removal and detail preservation suggests that smaller kernels are more appropriate for impulse noise removal if preserving image features is a priority.

The Mean filter, while computationally efficient, performed poorly in preserving image details. For a 3x3 kernel, it moderately reduced noise but caused noticeable blurring of edges. As the kernel size increased to 5x5 and 7x7, noise removal improved, but the image became overly smoothed, leading to a significant loss of sharpness and clarity. This behavior is consistent with the inherent limitations of averaging filters, which treat all pixels equally regardless of whether they are noise or part of the original image.

Gaussian filters produced mixed results. With a 3x3 kernel and σ=1, the Gaussian filter reduced noise while maintaining reasonable sharpness. However, it was less effective at removing impulse noise compared to the Median filter. Increasing the Sigma value to 2 resulted in greater smoothing, which blurred edges and fine details. Larger kernel sizes (5x5 and 7x7) compounded the problem by introducing excessive smoothing, making the image appear overly blurred and less detailed. These results highlight the limitations of Gaussian filters when dealing with impulse noise and reinforce the necessity of selecting filters based on the type of noise being addressed.

Umbaugh (2023) emphasizes the importance of preserving edges and features in applications such as medical imaging and remote sensing, where sharpness and clarity are essential for accurate analysis. The results of this study align with these principles, as the Median filter with a 3x3 kernel preserved edges and details far better than the other configurations, making it the most suitable choice for impulse noise removal.

Alignment with Preferred Methods

The findings of this study are consistent with established literature, which identifies the Median filter as the optimal choice for impulse noise removal. The superior performance of the Median filter with a 3x3 kernel can be attributed to its robustness against outliers, which allows it to effectively suppress noise without blending pixel values. This property explains why the Median filter outperformed the Mean and Gaussian filters, both of which rely on averaging mechanisms that incorporate noisy pixel values into the output.

The Gaussian filter, while effective for Gaussian noise, was less suited for impulse noise removal because of its weighted averaging mechanism. Similarly, the Mean filter, though computationally simple, failed to preserve edges and details due to its tendency to uniformly smooth the image. These results underscore the importance of tailoring filter selection to the specific type of noise in an image. As Farid (2004) and Bhuyan (2019) emphasize, understanding the nature of the noise is critical for selecting the appropriate filtering technique.

Conclusion

In conclusion, the Median filter with a 3x3 kernel proved to be the most effective configuration for removing impulse noise while preserving image features. This filter achieved the best balance between noise suppression and detail preservation, making it suitable for applications requiring high image quality and feature clarity. The findings align with theoretical insights from existing literature and demonstrate the importance of selecting filters and kernel sizes based on the specific noise characteristics and application requirements. These results highlight the value of the Median filter as a robust and reliable method for addressing impulse noise in digital images.

References

Farid, H. (2004). Fundamentals of image processing  https://farid.berkeley.edu/downloads/tutorials/fip.pdf

Bhuyan, M. K. (2019). Computer vision and image processing: Fundamentals and applications. CRC Press. https://www.perlego.com/book/1494377/computer-vision-and-image-processing-fundamentals-and-applications-pdf

Umbaugh, S. E. (2023). Digital image processing and analysis (4th ed.). CRC Press. https://learning.oreilly.com/library/view/digital-image-processing/9781000789911/